# Field Dynamics and Environment Modeling
Standalone Report for MSc Thesis:
“Modern Control Methods for Agricultural Irrigation”

Tara Torbati
ITMO University, Saint Petersburg, Russia

April 2026

# Study Site Description

The study site is a 6-hectare agricultural field located in Gilan Province, Iran (38.298​∘N, 48.847​∘E), in the humid subtropical coastal zone south of the Caspian Sea. Gilan is Iran’s primary rice-producing province, accounting for over 238,000 hectares of paddy fields [1]. The field exhibits a bowl-shaped topography with elevations ranging from 74 m to 181 m above sea level. A digital elevation model (DEM) was obtained from USGS Earth Explorer and processed in QGIS to extract a 10×13 grid of 130 computational agents, each representing approximately 462 m​2.

## Topological Graph Construction

The DEM was used to construct a directed graph encoding water flow topology. For each agent n, the normalized elevation was computed as γn=zn−zmin/zmax−zmin. Each agent sends lateral water flow to all eight-connected neighbors with strictly lower normalized elevation. The number of lower neighbors Nrn determines the agent’s drainage capacity. Agents with Nr=0 are classified as sink agents—local minima where water accumulates. On this bowl-shaped field, sink agents are concentrated near the center and represent areas at risk of waterlogging.

# Climate Data

## Data Source and Period

Daily meteorological data were obtained from the NASA POWER database for 2000–2025 at coordinates (38.30​∘N, 48.85​∘E), providing satellite-derived and reanalysis-based parameters at ∼50×60 km resolution. Thirteen variables were extracted: T2M, T2M_MAX, T2M_MIN, PRECTOTCORR, RH2M, WS2M, PS, T2MDEW, ALLSKY_SFC_SW_DWN, GWETROOT, and GWETTOP.

## Preprocessing

The dataset was filtered to April–October for each year, yielding 5,564 daily records. No missing values were found. One rainfall anomaly was identified (April 7, 2022: 375 mm) and replaced with the calendar-day average (1.2 mm). Two reference evapotranspiration values were computed for each day (Section 2.3).

## Reference Evapotranspiration

Two methods were compared:

Hargreaves equation [2]: ET0H=0.0023⋅Tmean+17.8⋅Tmax−Tmin0.5⋅Rs

FAO Penman-Monteith [2]:

ET0PM=0.408 Δ Rn−G+γ 900T+273 u2 es−eaΔ+γ 1+0.34 u2

Over the 25-year rice season, Hargreaves overestimated ET​0 by 34% (6.78 vs 5.02 mm/day, r=0.965). This is due to Gilan’s high humidity (∼64%) and low wind (∼1.40 m/s), which suppress evapotranspiration but are not captured by Hargreaves. Penman-Monteith was selected as the standard [2].

Comparison of Hargreaves and Penman-Monteith ET​0. Left: seasonal pattern. Right: scatter plot with regression.

## 25-Year Climatology

Mean temperature ranges from 12​∘C in early April to 26​∘C in mid-July. Precipitation shows a dry mid-season (June–August, 0.5–1.5 mm/day) flanked by wetter months. Solar radiation peaks at 26 MJ/m​2/day in June. Relative humidity remains 60–75% throughout—a defining feature of Gilan’s Caspian climate. Additional climate figures are provided in Appendix A.

25-year daily average temperature (April–October, 2000–2025).

25-year daily average rainfall (April–October, 2000–2025).

# Crop Parameterization

Two crops were selected: rice (Hashemi cultivar) and tobacco (Nicotiana tabacum). Both are commercially significant in Gilan [1, 3]. Their contrasting characteristics—high Kc/shallow roots/drought-sensitive rice versus moderate Kc/deep roots/drought-tolerant tobacco—enable evaluation of control algorithms under different agronomic conditions.

## Rice (Hashemi Cultivar)

### Base Temperature and Thermal Time

The base temperature Tbase=10∘C follows the standard GDD convention [4, 5] and the baseline model [6]. The AquaCrop convention of 8​∘C [20] produces proportionally higher GDD values without changing maturity dates.

The maturity threshold θ18=1250 ​∘C⋅day was calibrated from the 25-year dataset and validated against Sadidi Shal et al. (2021), who measured 1216–1352 GDD for Hashemi rice in Gilan [7]. With a 20-day nursery contributing ∼210 GDD (+5​∘C greenhouse boost), 20/26 years reach maturity by August 20.

### Season Window

Transplanting: May 20 (DOY 141), when 7-day rolling Tmean≥18∘C and Tmin≥12∘C [8, 9].

25-year average May temperature with transplanting thresholds.

Harvest: August 20 (DOY 233). WMO R20mm events (≥20 mm/day) [10] increase sharply after this date, causing lodging [10] and harvest disruption [11].

Extreme rainfall events (≥20 mm/day) per day of August, 2000–2025.

Field season: 93 days. Nursery provides x2,init=210 GDD. Other parameters: Kc=1.15, p=0.20 [2], θ5=400 mm, θ13=0.65 g/MJ [12, 13], HI = 0.42 [14]. Expected yield: 3500–4000 kg/ha (optimized) or ∼2000 kg/ha (traditional flooding, consistent with local farmer reports).

## Tobacco (Nicotiana tabacum)

### Base Temperature and Thermal Time

Tbase=10∘C, within the 10–13​∘C restriction range [15]. θ18=1200 GDD (field only), calibrated to match the 104-day season. Duration consistent with FAO (90–120 days) [16] and Britannica (100–130 days) [17]. x2,init=0 (nursery conditions unknown—seedlings delivered by company).

### Season Window

Transplanting: May 25 (DOY 146). Harvest: September 5 (DOY 249). 104 field days.

### Other Parameters

θ5=700 mm (FAO: 500–1000 mm) [16]. Kc=0.90, p=0.50 [16]. θ9=30∘C (FAO optimal ceiling) [15, 16]. θ13=0.30 g/MJ [18], HI = 0.55. Expected yield: 2000–2500 kg/ha [16].

## Parameter Summary

Complete crop and soil parameters.

| Parameter | Symbol | Rice | Tobacco | Source |
| --- | --- | --- | --- | --- |
| Shared soil parameters (silty loam) |  |  |  |  |
| Water uptake coeff. | θ1 | 0.096 | 0.096 | [6] |
| Wilting point (vol.) | θ2 | 0.15 | 0.15 | [2] |
| SCS threshold | θ3 | 10 mm | 10 mm | [6] |
| Drainage coeff. | θ4 | 0.05 | 0.05 | [6] |
| Field capacity (vol.) | θ6 | 0.35 | 0.35 | [2] |
| Crop-specific parameters |  |  |  |  |
| Root depth | θ5 | 400 mm | 700 mm | [2], [16] |
| Base temperature | θ7 | 10​∘C | 10​∘C | [5], [15] |
| Heat stress onset | θ9 | 35​∘C | 30​∘C | [16] |
| Extreme heat | θ10 | 42​∘C | 38​∘C | – |
| RUE | θ13 | 0.65 | 0.30 | [12], [18] |
| Drought sensitivity | θ14 | 0.80 | 0.60 | [6] |
| Maturity GDD | θ18 | 1250 | 1200 | [7], calib. |
| Max interception | θ19 | 0.95 | 0.90 | [6] |
| 50% intercept. GDD | θ20 | 417 | 450 | [19] |
| Crop coefficient | Kc | 1.15 | 0.90 | [2], [16] |
| Depletion fraction | p | 0.20 | 0.50 | [2], [16] |
| Harvest index | HI | 0.42 | 0.55 | [14], [16] |
| Nursery GDD | x2,init | 210 | 0 | Calib. |
| Initial biomass | x4,init | 60 g/m​2 | 8 g/m​2 | Est. |
| Season | – | DOY 141–233 | DOY 146–249 | This study |
| Field days | – | 93 | 104 | – |
| FC | – | 140 mm | 245 mm | θ6×θ5 |
| WP | – | 60 mm | 105 mm | θ2×θ5 |
| TAW | – | 80 mm | 140 mm | FC − WP |
| RAW | – | 16 mm | 70 mm | p× TAW |

# Water Budget and Scenario Selection

## Water Budget Formulation

Crop water demand: ETc=ET0PM⋅Kc⋅Ndays. Full irrigation need: Ifull=maxETc−P,0, using 25-year average rainfall (Method B).

Water budget summary.

|  | Rice | Tobacco |
| --- | --- | --- |
| Season duration | 93 days | 104 days |
| Avg ET​0 (PM) | 5.02 mm/day | 4.95 mm/day |
| Crop demand | 537.0 mm | 463.4 mm |
| Avg season rainfall | 53.3 mm | 74.5 mm |
| Full irrigation need | 483.7 mm | 389.0 mm |
| Budget at 70% | 338.6 mm | 272.3 mm |
| Budget at 50% | 241.8 mm | 194.5 mm |

Rice demand (537 mm) is consistent with Hashemi field measurements: 534 mm (SRI) to 632 mm (traditional flooding) [14]. Tobacco demand (463 mm) falls within the FAO range of 400–600 mm [16]. Despite a longer season, tobacco needs less irrigation due to lower Kc and higher rainfall.

## Scenario Selection

Selected climate scenarios.

| Scenario | Year | Rice (mm) | Tobacco (mm) |
| --- | --- | --- | --- |
| Dry | 2022 | 39.7 | 54.0 |
| Moderate | 2020 | 42.1 | 73.3 |
| Wet | 2024 | 176.8 | 163.2 |

# Agent-Based Model

The ABM follows López-Jiménez et al. [6]. Each of N=130 agents maintains: x1 (soil water, mm), x2 (thermal time, ​∘C⋅day), x3 (maturity stress), x4 (biomass, g/m​2).

## State Equations

Water balance:

x1nk​+​1=x1nk−φ1n−φ2n−φ3n−φ4,outn+φ4,inn+φ2,inn+Pk+unk

Thermal time: x2nk+1=x2nk+maxTmean−θ7,0

Maturity stress: x3nk+1=x3nk+θ111−h2+θ121−h3n

Biomass:

x4nk​+​1=x4nk+θ13⋅h3n⋅h6n⋅h7⋅gx2n⋅Rsk

## Water Fluxes

φ1n=minmaxθ1x1n−θ2θ5,0,ET0 (transpiration)

φ2=P−θ32/P+4θ3 if P>θ3, else 0 (SCS runoff)

φ3n=θ4⋅maxx1n+P−φ2−θ6θ5,0 (deep drainage)

φ4,outn=maxx1n−θ6θ5,0; φ4,inm=n∈Um​φ4,outn/Nrn (lateral flow)

## Stress and Growth Functions

h1=maxTmean−θ7,0; h2: linear between θ9 (no stress) and θ10 (full stress); h3n=1−θ14⋅max1−φ1n/ET0,0 (drought); h6n=clip1−x1n−θ6θ5/θ6θ5,0,1 (waterlogging); h7=1 if Tmean>θ7, else 0.

Growth function:

gx2n=θ19/1+e−0.01x2n−θ20x2n≤θ18/2θ19/1+e0.01x2n+x3n−θ18x2n>θ18/2

# Model Corrections and Extensions

Three corrections to the baseline [6] were implemented.

## Mass Conservation

The original water balance includes φ4,in but omits φ4,out, violating mass conservation. The corrected Eq. [eq:water] includes −φ4,outn.

## Drought Stress on Biomass

The original biomass equation lacks h3, so drought does not reduce biomass. The corrected Eq. [eq:biomass] includes h3n, reducing biomass by up to θ14×100% under maximum drought (80% for rice, 60% for tobacco). This is essential for irrigation optimization: without it, the controller has no incentive to irrigate.

## Surface Runoff Redistribution

Originally, φ2 leaves the system. A simple redistribution was implemented: each agent’s runoff flows to lower neighbors with immediate infiltration (φ2,inn). A cascading alternative (top-to-bottom SCS re-application) was also tested.

Runoff redistribution comparison, no irrigation, wet scenario (2024).

| Mode | Yield (kg/ha) | x1 sinks (mm) | x1 hills (mm) |
| --- | --- | --- | --- |
| None (original) | 2135 | 71.2 | 71.2 |
| Simple (A) | 2273 | 74.8 | 72.0 |
| Cascade (B) | 2278 | 79.5 | 72.0 |

Simple redistribution captures the first-order effect (6.5% yield improvement) with negligible difference from cascade (0.2%). Simple was adopted.

No-irrigation runoff mode comparison, wet scenario (2024).

## Biomass Calibration

θ13 was calibrated so x4 represents g/m​2 dry matter. Yield: Y=x4×HI×10 (kg/ha). Rice: validated against ∼2000 kg/ha (farmer reports) and 3500–4000 kg/ha (optimized) [14]. Initial conditions: rice x2,init=210, x4,init=60; tobacco x2,init=0, x4,init=8.

# Model Validation

## GWETROOT Cross-Validation

ABM soil water was compared to NASA MERRA-2 GWETROOT. Both normalized: ABM as x1−WP/FC−WP; GWETROOT as native 0–1 fraction. Pearson correlation assessed temporal pattern agreement.

Cross-validation: ABM vs NASA GWETROOT.

| Scenario | Year | Pearson r | p-value |
| --- | --- | --- | --- |
| Dry | 2022 | 0.738 | 3.1×10−17 |
| Moderate | 2020 | 0.534 | 3.4×10−8 |
| Wet | 2024 | 0.337 | 9.6×10−4 |

Normalized soil moisture: ABM (blue) vs NASA GWETROOT (brown).

The dry-year r=0.74 (p<0.001) confirms the ABM captures wetting/drying dynamics. Decreasing correlation with increasing rainfall is expected (reduced signal contrast in wet conditions). The absolute level difference (ABM ∼0.08 vs GWETROOT ∼0.46) reflects: (1) NASA’s deeper root zone (∼1000 mm vs 400 mm); (2) GWETROOT observes irrigated fields while the ABM simulates rain-fed conditions; (3) different soil parameterizations.

# References

SurfIran, “Gilan’s rice fields,” 2025. Available: https://surfiran.com/mag/rice-transplantation-in-gilan-province/

R. G. Allen et al., “Crop evapotranspiration,” FAO Irrig. Drain. Paper 56, 1998.

“Tobacco,” Encyclopaedia Iranica. Available: https://www.iranicaonline.org/articles/tobacco/

G. S. McMaster and W. W. Wilhelm, “Growing degree-days,” Agric. For. Meteorol., vol. 87, pp. 291–300, 1997.

S. Laohasiriwong et al., “High temperature alters phenology in rice,” Plants, vol. 12, no. 3, p. 666, 2023.

A. López-Jiménez et al., “Agent-based MPC of soil-crop irrigation,” Control Eng. Pract., vol. 150, 2024.

S. M. T. Sadidi Shal et al., “GDD of Hashemi rice in Guilan,” J. Climate Res., vol. 1400(45), pp. 143–152, 2021.

P. Paredes et al., “Base and upper temperature thresholds for GDD,” Agric. Water Manag., vol. 319, 2025.

R. Faghani et al., “Planting date and seedling age effects on rice yield,” Afr. J. Biotechnol., 2011.

A. A. Sarma et al., “Adaptation of wetland rice to extreme weather,” 2018.

S. Nayak et al., “Extreme rainfall indices and rice productivity,” 2011.

S. R. Patel et al., “RUE in irrigated rice,” Paddy Water Environ., vol. 11, pp. 477–486, 2013.

V. O. Sadras and R. Trenchero, “Radiation use efficiency,” in Crop Physiology, Elsevier, 2021.

N. Jalali et al., “Water requirement of Hashemi rice,” Water Res. Agric., 2021.

Y. Y. Li et al., “Cold stress effects on tobacco,” BMC Plant Biol., vol. 21, p. 131, 2021.

FAO, “Crop information: Tobacco.” Available: https://www.fao.org/land-water/databases-and-software/crop-information/tobacco/en/

“Tobacco,” Encyclopaedia Britannica, 1999.

R. Ferrara, “RUE in flue-cured tobacco,” Field Crops Res., 2002.

Y. Zhang et al., “LAI simulation of tobacco,” Chin. J. Eco-Agric., 2017.

R. S. Raoufi et al., “Climate changes and rice cultivation,” Int. J. Plant Prod., 2025.

TasteIran, “Iran rice harvesting.” Available: https://www.tasteiran.net/stories/10/iran-rice-harvesting

# Supplementary Climate Figures

25-year daily average solar radiation, April–October.

25-year daily average relative humidity, April–October.

25-year daily average wind speed, April–October.

25-year daily average surface pressure, April–October.

25-year daily average NASA MERRA-2 soil wetness, April–October.

Daily temperature in May for 2021–2025.

Spatial distribution of x1 at harvest, three runoff modes, wet scenario (2024).

Scatter plots: ABM vs NASA GWETROOT for three scenarios.